- APPENDIX 1

Contact Officer:	Mary Thompson (mary.thompson@oxfordshire.gov.uk )

Tel: 01865 815901

Division(s): Eynsham

ITEM PN5

PLANNING & REGULATION COMMITTEE – 19 MAY 2008

APPLICATION FOR THE EXTRACTION OF SAND AND GRAVEL WITH ASSOCIATED PROCESSING PLANT, SILT PONDS, CONVEYORS AND ANCILLARY WORKS. RESTORATION TO WETLAND/REEDBED AND FISHING. EXTRACTION OF BASAL CLAY TO FORM HYDROLOGICAL SEALS AND FOR THE PURPOSES OF RESTORATION ON SITE

Report by Head of Sustainable Development

Location			Land at Stonehenge Farm, Northmoor

Applicant 			Hanson Aggregates

Application No 		07/0111/P/CM

District Council Area 	West Oxfordshire

Introduction

- This is an application to extract 1.55 million tonnes of sand and gravel from land at Stonehenge Farm, Northmoor. The sand and gravel extracted would be transported by conveyor to the existing Stanton Harcourt Quarry processing plant.

- The Oxfordshire Minerals and Waste Local Plan (OMWLP) shows this land as having been resolved to be permitted subject to agreement to control minerals traffic. The agreement was never concluded and the application was withdrawn in 2003.

- Location (Plan 1)

- The proposed extraction site is located between the villages of Standlake and Northmoor in West Oxfordshire. It lies 1 kilometre (0.6 miles) to the east of Standlake, 1.2 kilometres (0.7 miles) to the south west of the centre of Northmoor and 3 kilometres (1.8 miles) south of Stanton Harcourt. It lies approximately 10 kilometres (6 miles) south east of Witney and approximately the same distance south west of Oxford.

- The conveyor is proposed to run approximately 2 km (1.2 miles) to the north to the existing Stanton Harcourt plant site.

- The Site and its Setting (Plan 2)

- The extraction site is currently farm land with parts being used for arable, set aside and pasture. Fields within the site are divided by hedgerows and the site contains small areas of woodland and a man-made duck pond.

- The extraction site is bordered by farmland and the industrial and residential units of Park Farm are to the north-east.

- The River Thames lies approximately 400 metres to the south of the southern edge of the extraction area and the River Windrush is adjacent to the western boundary of extraction.

- The extraction site is generally flat with a gentle fall towards the River Thames. A high voltage electricity transmission line crosses through the middle of the site in an east-west direction.

- The conveyor route passes under the Standlake Road which lies to the north of the extraction site. It also crosses over a bridleway and public footpath. Pinnocks Farm is the closest property to the conveyor at approximately 15 metres away.

- Northmoor footpath 2 leads from Standlake Road along the proposed route of the conveyor, which crosses agricultural land, into the proposed extraction site and through it. Northmoor footpath 12 is also partly within the site boundaries.

- There is a Scheduled Ancient Monument (SAM) nearby to the north and east of the extraction site and the conveyor route crosses the area covered by this designation. It is an area of archaeological remains that is believed to be the remains of a Romano-British settlement with origins in the Iron Age.

- The closest residential properties to the proposed extraction are Moreton House, Stonehenge Bungalow, Stonehenge Farmhouse, Park Farmhouse and Park Lodge. These all lie between 100 and 350 metres from the edge of extraction.

- Rose Cottage, Little Brook, Brook Field, Pinnocks Farm and Manor Farm all lie within 300 metres of the conveyor route.

- The existing processing plant has woodland to the west, silt ponds to the east and restored lakes following extraction to the north. There are some smaller restored lakes and agricultural land to the south and a bridleway runs along the southern boundary. The plant site has stockpiles of sand and gravel. The plant site access is onto Cow Lane to the west.

- Details of Application

- Extraction Site

- The proposed extraction would take approximately 8 years. Restoration would then be to wetland, reedbed and fishing and would take about eighteen months to complete. Clay would be dug from the bottom of the site and engineered to provide a reedbed habitat in order that there would be no need to import materials for restoration.

- The total site area is 59 hectares (146 acres), including extraction areas, soil storage bund areas, the conveyor, processing plant and silt disposal ponds.  Extraction would take place on 31.2 hectares (77 acres) of the site.

- The proposed working plan identifies 6 phases of extraction. Phases 1, 3 and 5 are proposed to be worked wet and 2, 4 and 6 are proposed to be worked dry through the use of an electric dewatering pump. A clay seal will first be formed around the sand and gravel to be worked to prevent drawdown of groundwater in the surrounding land in order to prevent any adverse effects from dewatering.

- Each phase would be stripped and the soils placed in storage mounds that are proposed would contribute to visual and noise screening. The sand and gravel would then be worked using hydraulic excavators and loading shovels.

- The footpath that crosses the extraction area would have to be diverted during extraction, and is proposed to be re-established on its original alignment on restoration.

- Excavated sand and gravel would be transported by conveyor for processing at the Stanton Quarry plant site.

- Conveyor and Processing Site

- The conveyor route would be fixed for the duration of the workings. A track would run adjacent to the conveyor for maintenance. It would travel through a tunnel under Standlake Road and safe crossing arrangements would be made for where the conveyor route crosses rights of way.

- The sand and gravel material would be washed and graded at the processing plant. A new silt lagoon would be created from an existing void at Stanton Harcourt  Quarry for the deposit of silt from the processing.

- Following processing the sand and gravel would be transported by lorry. Routeing of lorries is proposed across Cow Lane and through the Dix Pit complex, which is opposite Stanton Harcourt Quarry, then left on the B4449 towards Hardwick and then onto the A415 northwards towards the A40. This routeing would avoid the villages of Stanton Harcourt, Sutton and Standlake.

- It is anticipated that the development would give rise to an average of 40 lorry movements per day, spread throughout the day. The application states that this does not represent a change from the levels at Stanton Harcourt Quarry when it was operating. Stanton Harcourt Quarry ceased production in 2007.

- Restoration

- The extraction areas are proposed to be restored, as each phase is worked, to reed beds and open water. The areas that were worked wet will be restored to open water with graded slopes to encourage reed growth. Islands and shallows will be created in the areas that will be dewatered before extraction before the water levels rise using soils, clays from beneath the sand and gravel and overburden from within the site. It is not proposed to import material to create the restored landform. It is proposed that the margins would be planted with blocks of trees and shrubs. The processing plant and silt pond area will be restored to small water bodies with areas of indigenous tree and shrub planting. There would then be a five year aftercare period and 20 year management plan.

- Following discussions with a number of consultees, the restoration plan now incorporates an additional circular permissive path around the lake in the north west of the site and a small car park by this lake off the Standlake Road. This lake and the other areas to the west of the site will be restored to fishing and low key recreational use.

- Environmental Statement

- The application is supported by an Environmental Statement (ES.) This covers potential environmental impacts on landscape and visual amenity, ecology, soils, hydrology, cultural heritage, and the possible impacts of noise and dust. It considers alternatives to the development and mitigation measures.

- The section on noise provides a noise assessment considering the types of noise which may be generated and noise levels were predicted at noise sensitive properties. It was established that predicted noise levels are below the limits set out in Minerals Policy Statement 2. The predicted noise levels took into account mitigation measures incorporated into the development such as noise screen banks and the use of modern equipment. It concludes that noise impacts from extraction and processing would not be significant.

- The section on air quality identifies the main sources of dust as mineral extraction, materials handling, processing plant, haul roads and stripping and restoration. The assessment provides a risk rating for each of the sensitive properties and concludes that the impacts of dust emissions would be very low or insignificant. Mitigation and control measures such as the use of a water bowser to dampen down dust have been worked into the proposals.

- The consideration of landscape and visual impacts considers the landscape character of the area and the landscape impacts for all stages of development at the extraction area, processing plant and conveyor. A variety of impacts are identified, major and minor. The major visual impacts are found to affect the public rights of way which cross the application area. Measures to reduce the landscape and visual impact of the development are put forward, including the retention of screening vegetation around the site.

- Ecology was considered through a Phase 1 habitat survey, hedgerow assessment and protected species survey. Some signs of badger foraging were found on site but there was no direct evidence of protected species. The proposal retains the majority of field boundary habitats as these have a higher ecological value. The report suggests further mitigation measures such as a watching brief for the presence of protected species. It concludes that the habitats which would be permanently lost are adequately compensated in the long term by restoration habitat creation. A further ecological appraisal was submitted which included further surveys and found that the loss of the two semi-improved grassland fields that are relatively plant species rich would be compensated in the long term by the wetland habitats created by the restoration. The permanent loss of the species rich small pond is a permanent adverse ecological impact that would require compensation in the form of the creation of new ponds in the restored site.

- The quality of the agricultural land was assessed and it was found that most of the soils do not meet the definition of ‘best and most versatile agricultural land.’ The main limitations on the quality of soils is droughtiness and seasonal wetness. As the soil would be used to create reedbeds on restoration the report concludes that the soil resource would be effectively reused on site.

- A hydrological study includes mathematical modelling of groundwater flows which shows that with mitigation measures there would not be a significant adverse hydrological impact on the nearby aquifer, the SAM and nearby streams. These mitigation measures have been incorporated into the proposals. The flood risk assessment shows that there is no significant risk posed by this development.

- The impacts on cultural heritage were considered. The report assesses the impact on eight receptors and considers the effect on each. It concludes that there would be no significant impacts to listed buildings or other historic buildings in the Northmoor conservation area. There would be a moderate adverse impact on a small Iron Age site and a moderate to major adverse impact on archaeology of the Romano-British period. A small part of the scheduled Roman site would be subject to a severe adverse impact by drying out caused by groundwater draw down from dewatering.   Mitigation would reduce the impacts on cultural heritage. A further section on the proposed hydrological protection of the SAM by the use of clay seals was submitted and sets out the mitigation measures that would be implemented. It suggests an annual review meeting to review these mitigation measures.

- Consultations

- The application was originally sent out to consultation in early 2007. However, the consultation responses that were received made it clear that more information was needed about various aspects of the development. In January 2008 additional information to accompany the application and a revised Environmental Impact Assessment was submitted following discussions between the applicant and relevant consultees. There was a further consultation period in order to get views on the additional information. The responses reported below refer to the most recent consultation responses that have been received after the new information was submitted.

- West Oxfordshire District Council:

- Planning - Strong objection. The conclusions reached in the hydrological/flood risk assessment appear to be based on modelling that fails to take account of the realities of the 2007 flood event. The applicant should show the probable error range and sensitivity of its predictions. There are a series of material considerations which would justify the refusal of this application. The Oxfordshire Minerals and Waste Local Plan (OMWLP) is now dated and does not reflect the current emphasis on recycled materials. To allow a major new supply of minerals would undermine efforts to increase recycling. Questions the need for the aggregate given the downturn in the economy. Decisions should be made as the result of strategic analysis not in a piecemeal manner. The HGV traffic resulting from this development would have a substantial impact on roads which are already congested. The bridge over the River Thames at Newbridge has been declared unsafe for vehicular traffic and this will add to road miles and congestion. Any decision on this application prior to a decision on the replacement of this bridge would be premature. The County Council has accepted the need for a ‘cumulative impact test’ to be used when assessing the impact of mineral working. Northmoor is almost entirely surrounded by old minerals workings. This application does not meet the criteria set out in OMWLP policy PE1. Mitigation proposals set out in the reports in the Environmental Statement require high levels of monitoring. A precautionary approach would be to refuse the application to avoid the need for costly open-ended monitoring.

- Environmental Health - West Oxfordshire District Council Environmental Health Officer requests conditions to cover dust, noise and domestic water supplies. Suggests suitable conditions for a dust management scheme and noise scheme with a noise limit of 55 dB(A) Leq 1 hour (free field) at the boundary of the site. Concerned about unlicensed private water supplies and suggests a condition requiring that the aquifer is not polluted and suggests that a condition could be considered to require the developer to supply temporary or permanent alternative water supply to affected residents in the event that the aquifer becomes polluted or if water flows become insufficient. Second response suggests conditions for a 51 dB(A) Leq 1 hour noise limit for phase 3, and 46 dB(A) Leq 1 hour, free field for other phases.

- Northmoor Parish Council - Object. Acknowledge the submission of supplementary information. However this information cannot counter the cumulative effect of many years of gravel extraction in the Lower Windrush Valley. The development is described as an extension, which is not accurate. The demand for the gravel is in areas to the south of the site yet the route over Newbridge has a weight restriction. It is proposed to route traffic along the A415 however this road was not intended for such purposes and has a poor accident record. The conveyor is not acceptable especially where it crosses the Standlake Road, as it would follow a delightful country walk and the crossing of the road would cause noise and dust close to homes. Concerned about flooding, especially since the July 2007 flooding. Changes to the groundwater may well lead to an increased risk of flooding. The bunds would require maintenance in perpetuity. Application gives little reassurance to those concerned about water contamination or deprivation in wells. There has not been proper consultation on the afteruse.

- Stanton Harcourt Parish Council - No response at time of writing the report.

- Standlake Parish Council - No response at time of writing the report.

- Environment Agency – Draft Response

- Groundwater – Extraction of sand and gravel at this site would pose a significant risk to the water environment. Without mitigation there is a risk that licensed water supplies and private wells might dry up and the River Windrush and other watercourses would be at considerable risk. At present the Environment Agency cannot control dewatering activities and they may only be controlled by placing conditions on the planning permission. Therefore, conditions are suggested. Consider it inappropriate to permit dry working of phases 5 and 6. Recommend that prior to the workings of phases 5 and 6 a complete hydrological review and re-assessment of working methods is completed. The review should evaluate the reliability of the groundwater model in the light of the monitoring collected during workings, give consideration to recalibrating and re-running the groundwater model and recommend changes to the working scheme. We recognise that the wet working of phases 5 and 6 would compromise the restoration scheme but are confident that the hydrological study already carried out shows that, with the mitigation proposed, it will not be necessary to work the final stages wet. Water supplies have already been severely affected by drought in the past with several wells drying up diurnally in 1997. Therefore it is not incumbent on the applicant to guarantee the supplies, but to provide mitigation which is sufficient to ensure no increase in risk to them. Support the applicant’s intention to protect the River Windrush by working phases 1, 3 and 4 wet and would recommend a condition be imposed to that effect. Suggests a condition for the submission of a contingency scheme to be implemented if there is insufficient dewatered groundwater to maintain recharge trenches. Suggests a condition for the locations of discharge points into the surface watercourses. Suggests standard conditions and advice.

- Nature Conservation: Suggests a condition for the retention of the pond as ponds have recently been added to the UK Biodiversity Action Plan (BAP) and this pond meets 2 of the 5 criteria for ‘priority ponds.’ There should be no working within 16 metres of main river watercourses. However restoration work to provide ecological linkage between the river corridor and the final restoration may be permitted subject to further discussions and necessary consents. A scheme to manage water levels to assist with the establishment and management of reedbeds should be proposed under a condition. Support the establishment of reedbed. However due to excavation of basal clay to form the reedbed islands and marginal shelves needed, the channels between the islands will be fairly deep and not as ecologically productive as shallower areas would be. These channels may be less than optimal and may limit the potential for the overall complex to support a wide range of flora and fauna. Lake margins should have a minimum of trees and scrubs so that rough grassland can establish to favour use by water voles and barn owls. Regarding the afteruse as a fishery, consent would be needed for stocking with fish. Given the location within the floodplain, would advise a low intensity fishery as there would be a greater risk of escape during flooding if there were high stocking densities.

- Flood risk – Do not have cause to raise objection to this development. Suggest a number of conditions to ensure that the development is carried out in accordance with the principles in the flood risk assessment and to prevent increased risk of flooding.

- Natural England - Previous consultation responses requested that no decision be made until protected species surveys had been conducted and that more sustainable use of soils be explored. The second consultation response states that the survey work in respect of protected species is acceptable. The existing pond is a UKBAP priority habitat and therefore this pond should be retained if permission is granted. The application area comprises 27% soil classified as ‘the best and most versatile’ agricultural land. Some of this soil resource is being used as fill to create the landform. Whilst recognising that it may not be practicable to restore the site to a dry afteruse, Natural England expects the applicant to demonstrate that they have considered opportunities for using the soils in a more sustainable way than that proposed.

- RSPB - No objection. Strongly support the restoration to reedbed. There is an urgent need to replace reedbeds that are under threat in East Anglia due to sea level rise. The lack of inert fill does compromise the quality of the restoration, but the applicant has made every effort to design the best reedbed with the site constraints. Have some concerns about the loss of the pond and recommend a condition to retain it. If this is not possible there should be a condition requiring the creation of additional pond areas based on advice from pond conservation. A condition should be imposed requiring the retention of the black poplar tree near the pond and a sufficient stand off for its survival. The water control measures detailed on page 52 of part 2 of the Environmental Statement should be retained as part of the restoration for the long term management. It may be necessary to have the ability to manage water through pumps during reed establishment. Recommends that plans are modified to show no additional tree or scrub planting as this will be detrimental to key wetland species. Recommends that the Standlake road tunnel be made safe but retained as a bat roost after works. This should be considered in the design. Supports the provision of low key public access.  No objection to shooting on the site provided it is carried out in a sustainable manner. Recommends a condition requiring a refuge area. Supports the proposal for a 20 year aftercare agreement.

- BBOWT - The pond on the site should be preserved in situ as it is a priority habitat. Have not seen evidence that there is an overriding need to work minerals from this part of the site. Would also like to see the creation of further ponds as part of the restoration scheme. Supportive of the restoration to reedbed but would have preferred to see a mosaic of habitats including grassland. Recommend use of planning conditions/obligations to secure long term management of the site. Given the location of the development in the Lower Windrush Conservation Target Area (CTA) would welcome the requirement for the minerals extraction to contribute to the wider objectives through a similar mechanism to that employed for Standlake Common Nature reserve, i.e. a ‘per tonne’ levy for nature conservation within the CTA.

- Pond Conservation - Ponds are a Biodiversity Action Plan (BAP) priority habitat. Good quality ponds, such as the one that exists on this site, are now considered a ‘priority habitat.’ The existing pond should be retained. Creation of additional high quality ponds should be included in the restoration proposals. This is supported by Oxfordshire Structure Plan (OSP) policy EN2 and by PPS9.

- Defence Estates - No objection. Concerned about large areas of open water being created in phases 1, 3 and 5 of the restoration scheme. Confirms that there have been minor changes to the restoration scheme following the original submission of the application and Defence Estates original comments. No objection subject to conditions to ensure that the reed bed is not exposed and for fencing to prevent terrestrial access to waterfowl. Requests a bird management plan that allows the MOD monthly reports of bird species and numbers if requested and yearly access to the site for inspections. It should also prevent the breeding and roosting of certain bird species, and populations of certain species should be dispersed at the reasonable request of the MOD.

- English Heritage - When originally consulted had concerns that issues relating to the effect upon the groundwater regime and its impact on the SAM needed to be resolved before any decision was taken. Having considered the additional material can now confirm that the applicant is proposing to take steps to ensure that the SAM is not adversely affected. Recommends a condition for the submission of a ground water monitoring scheme. Subject to this condition the issues relating to groundwater monitoring are satisfactorily resolved.

- National Grid - No objection. This is a moderate risk development, encloses information to the applicant to help manage this risk.

- Southern Electric - No response at time of writing report.

- Thames Water - No objection. Suggests a trade waste effluent informative.

- County Ecologist - There are no statutory or non statutory wildlife sites directly or indirectly affected by the proposed development. No objection subject to the resolution of the following issues:

• 	No objection from Natural England regarding protected species.

• 	Opportunities for safeguarding existing pond feature/black poplar and establishing a new pond complex should be explored in consultation with Pond Conservation.

• 	Opportunities for improving upon existing proposed restoration scheme should be explored. Would particularly like to address the removal of shrub species on land bordering the lakes/reedbeds, improving the design of the open water lakes and the silt beds to benefit biodiversity.

• 	The permissive footpath/car park area should be designed to accommodate wheelchair access and advice should be sought from the Council’s Access Officer. The line of the path should be designed to provide maximum amenity benefit to the general public. The permissive use of the path should run for a period of 20 years beyond the end of the 5 year after-care period

• 	A hide should be installed at a suitable location adjacent to the existing public footpath and overlooking the wetlands/reedbeds.

• 	There should be a legal agreement requiring the submission of a long-term management plan covering a period of 20 years over and above the 5 year after-care period. Provision must be made for covering the costs of implementing this management plan over the 20 year period. There will need to be a financial contribution towards the wider aims and objectives of the Lower Windrush Valley Project.

- Archaeology - No objection. Largely satisfied that the cultural heritage elements of this proposal are acceptable and pleased to see that comments made in earlier consultation have been taken into account. Have some further comments regarding inaccuracies in the Environmental Statement text. Additional archaeological information has been submitted by a local resident as evidence opposing this application. This evidence is considered in detail and account taken of the objector’s views. However there is a paucity of supporting evidence and the specific interpretations made cannot be supported. English Heritage must be satisfied that any impacts on the SAM are covered by the SAM consent and that there is adequate mitigation. Support the proposal to carry out an archaeological monitoring strategy throughout the life of the extraction programme. Suggest conditions to ensure that development does not take place without a staged programme of archaeological mitigation measures.

- Transport Development Control - No objection in principle. Will require a legal agreement to construct and maintain the culvert beneath Standlake Road. There should be no extraction until this is in place. There should also be a routeing agreement to continue the routeing obligations currently in place for the processing plant.

- Rights of Way - The application makes provision for all rights of way. however there is a lack of proper detail concerning the crossing points on the path. Would like to see further information. A diagram is provided on the crossing point on the bridleway. However the width may be restrictive for those anymore. It should be widened, or perhaps the applicant could provide mounting blocks at both ends. Would like clarification regarding whether the conveyor crossing under the bridleway will be covered or not. It should be as it is likely to cause distress to horses to see the conveyor beneath them. Would like further information about the surfacing on the bridleway crossing and confirmation that vegetation at the bridleway crossing will be maintained at a reasonable height. The application requires the diversion of Footpath 2a Northmoor whilst mineral extraction and restoration take place in phase 4. The applicant should be advised that it can take up to eighteen months to secure a diversion and they should contact the rights of way team as soon as possible to begin the application process.

- Representations (Annex 3)

- It would be good to briefly highlight the main topic areas of concern to people here.

- Relevant Planning Policies (Annex 2)

- The key policies are those related to the need for the development, transport and impacts on the local environment.

- Need for the Development

- Government policy as set out in MPS1 states that Mineral Planning Authorities should provide for the maintenance of a landbank of at least 7 years extraction for sand and gravel. It states that if the landbank is below this level then additional reserves will need to be permitted if acceptable planning applications are submitted. Policy M3 of RPG9 also states that there should be a seven year landbank.

- OMWLP policy PE2 states that permission will not normally be given for mineral working outside the areas identified in the plan, unless the development is a small extension to existing sand and gravel quarries, or, the proposal satisfies other policies in the development plan and the apportioned supply cannot be met from the areas identified.

- Traffic

- OSP 2016 policy T8 states that proposals for development should be permitted only if they provide adequate access and mitigation of adverse transport impacts.

- OMWLP policy SH2 states that planning permission will not be granted for mineral extraction where the development would lead to a significant increase in traffic in Sutton, unless the Sutton bypass has been constructed and brought into use.

- OMWLP policy SH3 states that the County Council will seek routeing agreements with operators in order to limit the use of the A415 through Standlake and south over Newbridge. The preferred routes will be the A415 north of Standlake to the Ducklington bypass or the B4449 via the Blackditch, Sutton and Eynsham bypasses.

- Flood Risk and Groundwater

- OSP policy EN9 states that development in areas at high risk from flooding or in the functional floodplain will not be permitted. New development should not lead to an increase in run off which would increase flooding elsewhere.

- OSP policy EN8 states that development that would lead to an unacceptable deterioration in water quality will not be permitted.

- PPS25 classifies sand and gravel working as a ‘water compatible development.’ Therefore it is not prohibited from taking place in the floodplain.

- West Oxfordshire Local Plan (WOLP) policy NE8 states that new development will not be permitted within areas at risk of flooding if it is likely to impede the flow of water, result in the net loss of floodplain storage or increase the risk of flooding elsewhere.

- Ecology

- PPS9 key principle (vi) states that planning decisions should aim to prevent harm to biodiversity. When granting planning permission that would result in significant harm to biodiversity conservation interests, local planning authorities will need to be satisfied that the development cannot reasonably be located on any alternative sites that would cause less or no harm. In the absence of alternatives adequate mitigation measures should be put in place before permission is granted. Where a planning decision would result in significant harm that could not be mitigated against, appropriate compensation measures should be sought. If that significant harm cannot be prevented, adequately mitigated against, or compensated for, then planning permission should be refused.

- Regional Planning Guidance for the South East (RPG9) policy E2 states that the region’s biodiversity should be maintained and enhanced with positive action to achieve the targets set in national and local biodiversity action plans through planning decisions and other measures.

- PPS9 paragraph 11 states that local authorities should conserve important natural habitat types that have been identified in the Countryside and Rights of Way Act 2000 section 74 list. These correspond with the list of UK BAP priority habitats. The updated list is now under section 41 of the Natural Environment and Rural Communities Act 2006.

- OSP paragraph 5.8 states that planning authorities can make significant contributions to the BAP process through appropriate use of conditions and management agreements. The restoration and management of mineral workings can strengthen the status of priority habitats.

- OSP policy EN2 states that sites which support specially protected species will be protected from damaging development. On other sites of acknowledged nature conservation importance development will be permitted only if there is an overriding need or if damage to the ecological interest can be prevented by the use of conditions or planning obligations.

- OMWLP policy PE14 states that sites of nature conservation importance should not be damaged. Proposals which would affect a nature conservation interest will be assessed by taking into account the importance of the affected interest; the degree and permanence of the projected damage and the extent to which replacement habitat can be expected to preserve the interest in the long term.

- OMWLP policy SH5 states that afteruses for the Stanton Harcourt area should normally conform with those on the Proposals Map. Planning permission will not normally be granted until these afteruses and means of funding them have been secured. The Proposals Map shows the Stonehenge Farm site as having a nature conservation afteruse.

- OMWLP policy SH6 states that the County Council will seek the establishment and long term management and maintenance of nature conservation areas and areas of general public access and planning permission will not normally be granted until the means of funding these have been secured. OMWLP paragraph 7.39 states that a warden would be responsible for co-ordinating area-wide management.

- WOLP policy NE13 states that priority habitats and species will be maintained and enhanced. Development proposals should include measures to mitigate any effects on features of nature conservation importance, including, where appropriate the provision of compensatory habitats or management.

- WOLP policy NE15 states that development that would have an adverse effect on a site supporting specially protected species will not be permitted unless damage to the ecological interest can be prevented through compliance with conditions or planning obligations.

- Archaeology

- OSP policy EN6 states that there will be a presumption in favour of preserving in situ nationally and internationally important archaeological remains and their settings. Development affecting other archaeological remains should include measures to secure their preservation in situ, or where this is not feasible, their recording or removal to another site.

- PPG16 states that the desirability of preserving a national monument and its setting is a material planning consideration whether or not it is scheduled. There should be a presumption for preserving their physical structure in situ.

- WOLP policy BE12 states that development proposals which adversely affect the site or setting of archaeological monuments of local or national importance, whether scheduled or not, will not be permitted. WOLP policy BE13 states that applicants may be asked to provide an archaeological assessment and/or field evaluation prior to the determination of applications affecting areas of sites archaeological potential.

- Landscape Character

- OSP EN1 states that development proposals should contribute to the protection, maintenance and where possible, enhancement of Oxfordshire’s landscape character and should only be permitted if it does not unacceptably damage the local landscape.

- WOLP policy NE1 states that proposals for development in the countryside should maintain or enhance the value of the countryside.

- WOLP policy NE3 states that development will not be permitted if it would harm the local landscape character. When considering proposals in West Oxfordshire Landscape Assessment, the Lower Windrush Valley Project Report will be taken into account where appropriate.

- WOLP policy NE6 states that planning permission will not be granted for proposals that would result in the loss of trees, hedgerows, woodlands or their settings which are important for visual, historic or biodiversity value. Removal will only be permitted where it can be demonstrated that development would enhance the landscape quality and nature conservation of the area.

- WOLP policy BE5 states that to special architectural, historic and environmental character or appearance of conservation areas will be preserved or enhanced. Every effort will be made to ensure that the character or appearance is not eroded by the introduction of unsympathetic development proposals either within or affecting the setting of the designated area.

- Amenity

- OMWLP policy PE18 states that in determining applications the County Council will have regard for the provisions in the Code of Practice in the OMWLP. The Code of Practice covers amenity issues such as hours of working, noise and dust.

- WOLP policy BE19 states that permission will not be granted for development that would create noise that would expose housing and other noise sensitive development to significant noise disturbance, unless there is an overriding need for the proposal which cannot be met elsewhere.

- OMWLP PE3 states that appropriate buffer zones will be safeguarded around mineral working sites for protection against unacceptable losses of residential or natural amenity. Paragraph 4.8 of the text of the plan suggests that appropriate buffer zones would be 350 metres from villages and hamlets and 100 metres from individual dwellings.

- Other Issues

- OSP R2 states that development that would damage the rights of way network will be resisted.

- OMWLP policy PE12 states that in appropriate cases general public access will be sought to restored mineral workings; where this is required, planning permission will not be granted until its provision and long term management has been secured.

- Comments of the Head of Sustainable Development

- The key issues to consider are need, traffic, flood risk, ecology, archaeology, landscape character and amenity. Issues raised in representations are considered at Annex 3.

- Need for Sand and Gravel

- The landbank in Oxfordshire was estimated to be 2.6 years in February 2008. This is based on the need to meet an apportionment of 1.82 million tonnes of gravel per year. The 2.6 year estimation figure is well below the 7 year landbank required by RPG9 policy M3 and MPS1.

- With respect to OMWLP policy PE2, I do not consider this proposal to be a small extension relevant to policy SD2 because although it utilises an existing plant site it does not adjoin any existing workings. In addition, working is proposed for a period in excess of 3 years required to meet policy SD2. However, if the remaining areas in the Oxfordshire Minerals and Waste Local Plan were given permission the landbank would only increase by a further 0.7 years and there would still be a strong need for further permissions to be granted. Therefore, if this proposal satisfies other development plan policies, it would accord with OMWLP policy PE2, as it is clear that the apportioned sand and gravel cannot be met from within the areas identified in the plan.

- There is to be a review of government guidelines which may alter the amount of sand and gravel that Oxfordshire has to provide. The results of this review will not be finalised until 2009 at the earliest and in the mean time applications must be determined in line with existing policy. Even if the review did lead to a decrease in Oxfordshire’s apportionment, there would still be a substantial shortfall, given the current very low level of reserves with planning permission for extraction.

- There is a strong need for further permissions to be granted for sand and gravel extraction in Oxfordshire. Therefore, if the proposals are acceptable in terms of other policies in the development plan, then both MPS1 and OMWLP policy PE2 state that permission should be given.

- The WODC response states that there is a much greater emphasis on recycling since the OMWLP was adopted and to allow this development would undermine the drive to recycle. WODC also state that given the economic downturn they would question the need for the aggregate. However, the apportionment of sand and gravel for Oxfordshire takes into account the contribution of recycled materials and there is still a requirement for Oxfordshire to meet its landbank demands and this requirement is in up to date plans such as RPG9.

- Traffic

- The development does not propose any increase in traffic levels from the processing plant site above the levels from the existing operations at Stanton Harcourt Quarry. It is proposed to transport extracted material by conveyor to the processing site and that coupled with the fact that there are no proposals for the importation of material as part of the restoration scheme mean that there would not be any traffic impact on the narrow lanes around the extraction site itself, or in the village of Northmoor.

- There has been no objection from transport development control subject to legal agreements for the conveyor tunnel and for the routeing of vehicles leaving the processing site. Routeing is proposed by the applicant and would be in accordance with OMWLP policies SH2 and SH3. This would mean that heavy goods vehicles would not be routed through Sutton village or over Newbridge.

- There have been representations made that state that the applicant can not be trusted to comply with routeing agreements as they have not been complying with the agreement that currently covers vehicles leaving the quarry site and this has led to lorries travelling down restricted routes. A survey carried out by the County Council confirmed that lorries from Stanton Harcourt Quarry were ignoring the routeing agreement and travelling through Sutton. These lorries cause a nuisance in villages such as Sutton. It is not acceptable to permit a development that would lead to large numbers of HGVs travelling through local villages. Therefore, the routeing agreement is essential and is sought by OMWLP policy SH3 and required by OMWLP SH2. It is proposed that a legal agreement should include provision for periodic traffic surveying to the monitoring of the legal agreement, at the applicant’s expense. This monitoring would ensure that the agreement is complied with and that vehicles do not travel on inappropriate routes.

- The response from WODC states that it would be premature to make a decision on this application prior to a decision on whether to replace the bridge at Newbridge. The bridge now has a weight restriction on it and so could not be used by gravel lorries. A route is proposed that does not include the bridge and this route is considered acceptable to transport development control. Therefore the decision on whether to replace is bridge is not relevant to this application.

- The proposed development therefore accords with OSP policy T8, and OMWLP policies SH2 and SH3, subject to a routeing agreement with monitoring arrangements.

- Flood Risk and Groundwater

- This proposal is supported by detailed hydrological and flood risk assessments. The Environment Agency, the statutory consultee on flooding, has responded that if the development is carried out in accordance with the principles in the flood risk assessment then an increase to the risk of flooding will be prevented. Therefore, subject to conditions as recommended by the Environment Agency, to ensure that the development is carried out in a satisfactory way, the proposal accords with development plan policies related to flooding, including OSP EN9 and WOLP policy NE8.

- The hydrological assessment has also been accepted by the Environment Agency. Although the development does have the potential to pose a significant risk to the water environment, this risk can be reduced through mitigation. Planning conditions can be used to control the dewatering. The Environment Agency’s suggested condition to decide whether phases 5 and 6 should be worked wet or dry following a hydrological review would provide a further opportunity for the development to be carried out in a manner which avoids impact on the groundwater. Therefore the proposals accord with OSP policy EN8 in that they would not lead to an unacceptable deterioration in water quality.

- The WODC response states that the conclusions reached in the hydrological/flood risk assessment appear to be based on modelling that fails to take account of the realities of the 2007 flood event and the applicant should show the probable error range and sensitivity of its predictions. However the Environment Agency has not objected to this development after consideration of the work on flooding.

- Ecology

- There are no designated statutory or non-statutory sites which will be affected by the proposed development and Natural England have no objection regarding protected species. Therefore this development accords with OSP policy EN2 and does not conflict with WOLP policies NE13 and NE15.

- Thames Valley Environmental Record Centre (TVERC) has confirmed that the semi-improved grass fields within the application area do not qualify as UK BAP priority habitat. Therefore, this development would not cause significant harm to biodiversity and accords with PPS9 key principle (vi) without the need to further consider alternative sites or mitigation.

- This development would contribute towards RPG9 policy E2 because the restoration proposals would enhance the biodiversity of the site and therefore contribute towards achieving biodiversity action plan targets, through the creation of reedbeds, which are a UKBAP priority habitat. Restoration to reedbeds accords with the advice in OSP paragraph 5.8 which encourages contributions to the BAP process through mineral restoration and management. Restoration to reedbeds also accords with OMWLP policy SH5 as restoration would be to a nature conservation use.

- The only BAP priority habitat within the site is the existing pond. The applicant has agreed to retain this pond and a rare black poplar nearby. Therefore, subject to a condition requiring the safeguarding of this pond, the proposals  accord with paragraph 11 of PPS9 as an existing important natural habitat type is to be conserved.

- The application site is not a site of nature conservation importance. The nature conservation interest that does exist would be affected over the period of the development. However, the use of conditions, for example to retain the existing pond, could help to protect the most valuable aspects of the existing interest. In the longer term a replacement habitat for the rest of the site would be provided that would give greater interest, as reedbeds are a priority habitat and their design is capable of providing good quality benefits for BAP priority species including bittern, water vole and otter. The Environmental Statement and the application consider the ecological interest of the site. Therefore, the interest has been assessed taking into account the importance of the affected interest; the degree and permanence of the projected damage; and the extent to which replacement habitat can be expected to preserve the interest in the long term. The development accords with OMWLP policy  PE14.

- The applicant is willing to enter into a legal agreement to manage and fund the site for nature conservation for a 20 year period following the end of aftercare and make contributions towards the wider aims of the Lower Windrush Valley Project. Therefore, in the long term this development is likely to result in biodiversity gains and accords with OMWLP policies SH5 and SH6 relating to nature conservation and the means of funding them.

- The silting area was appraised by an ecologist for the applicant in April 2008. The ecologist submitted a letter that confirmed that areas of the pit are being used by a colony of Sand Martins. Therefore, any preparation of the pit to receive silt should not take place between March and August to avoid damage to nesting birds. This restriction could be added as a condition. The ecologist also recommends making provision for new sand martin habitat, which could also be required through condition. There were no protected species issues on the silting site.

- The applicant has agreed to provide a bird hide in the detailed restoration proposals. The County Ecologist asked for details of the shooting at the site to be included in the bird management plan, so that shooting can be sensitively managed.

- Archaeology

- The application site is located close to a Scheduled Ancient Monument (SAM). There is the potential that the SAM could be damaged by changes to the groundwater due to dewatering. However, English Heritage are satisfied that the proposals put forward do not put the SAM at risk. A condition for a programme of archaeological work would ensure that any archaeological remains are dealt with appropriately. This approach is acceptable to the County Archaeologist. Therefore, this development accords with OSP policy EN6 and WOLP policy BE13 and BE12.

- Landscape Character

- The development would change rather than damage the landscape character of the area. The restoration proposals would provide an acceptable alternative landscape. Therefore, the proposal does not conflict with OSP policy EN1. The restored site would maintain the value of the countryside because it would be of nature conservation value. Therefore, the proposals do not conflict with WODLP policy NE1.

- The landscape and visual assessment that forms part of the ES considers the setting of the site within the Lower Windrush Valley Project Area and the impact of the development on this landscape. Although there will be impacts on the landscape and visual amenity, the extraction would be temporary and would not affect the whole site at the same time. Following restoration, the development aims to enhance the landscape by improving visual interest. Issues such as visual amenity are inevitably subjective, but there is no reason that an area of reedbeds cannot be as visually pleasing as an area of agricultural fields. Therefore, the development does not conflict with WODLP policy NE3.

- Many of the existing areas of hedgerow and woodland are proposed to be retained. Those areas which are to be removed are not of ecological significance. The vegetation that is most important as a visual screen, for example that around Park Farm, is proposed to be retained. Therefore these proposals accord with WOLP policy NE6.

- The extraction area would be 450 metres from the edge of Northmoor Conservation Area and it would be screened by existing vegetation. The conveyor run and its crossing under the Standlake Road would be closer and therefore would be more likely to have an affect on the setting of the Northmoor conservation area. However, these would be temporary features with limited visual impact and would not affect the special character of the conservation area in the long term. They would not be visible from the village centre. Therefore, this development accords with WOLP policy BE5.

- Amenity

- The ES considers the potential impacts on amenity. There has been no objection from the Environmental Health Officer (EHO) and noise limits can be imposed through condition to ensure that the levels recommended are adhered to. The properties at Park Farm lie closest to the proposed extraction. However, extraction would not take place in the area closest to these houses for very long and the EHO has recommended a suitable noise limitation for phase 3, which could be imposed by condition.  Therefore, subject to these conditions, the development accords with OMWLP PE18 and the Code of Practice, regarding noise and also with WOLP policy BE19.

- Dust has also been a matter of concern amongst those who made representations on this application. However, dust was also considered in the Environmental Statement and showed that the impact of dust would be low and could be mitigated. There has been no objection from the Environmental Health Officer. Therefore, subject to the appropriate conditions, there is no reason to believe that this development would cause an unacceptable dust nuisance. Therefore, in relation to dust, the development accords with the Code of Practice referred to in OMWLP PE18.

- Residential properties within the Park Farm complex are closer than the 100 metres suggested as a guide in the text relating to OMWLP policy PE3. Other properties are all in excess of 100 metres buffer zone to the proposed extraction area from the extraction. Indeed the next closest small group of residential dwellings to the east of the site on the Standlake Road are further than 500 metres from the extraction area. However OMWLP policy PE3 requires appropriate buffer zones between houses and workings and the Environmental Statement has shown that the impact of the development on nearby houses can be successfully mitigated and there has been no objection to the proposed extraction boundaries from the Environmental Health Officer. Therefore the buffer zones provided are considered to be appropriate in this case and the development accords with OMWLP PE3.

- There are also properties that lie close to the proposed conveyor run, such as Pinnocks Farm and Manor Farm. The impacts of the conveyor would not be as great as the impacts of extraction. The possible impacts are considered in the Environmental Statement. It is unlikely to cause a noise nuisance as long as it is properly maintained, which can be required by condition. The ES states that the conveyor is virtually dust free when compared with the use of dump trucks for transporting mineral to the processing plant. Therefore, the proposed operation of the conveyor accords with policies protecting amenity, including OMWLP PE18 and OMWLP PE3.

- The processing plant has been operating for a number of years and there have been no complaints. It is isolated from residential properties. There are unlikely to be any amenity issues related to its continued use.

- Other Issues

- This development would require the temporary diversion of a footpath and its reinstatement after the development finishes. The footpath would run between areas of reedbed rather than through open fields. Although this is a change to its setting it could not be said to be damaging to the footpath and is therefore not contrary to rights of way policies such as OSP R2. There have been comments that horses would not use a crossing over a moving conveyor. The detail of the crossing could be determined in consultation with the rights of way officer who has suggested that a wider crossing, with the conveyor covered for two metres on either side, would cause less distress to horses and enable them to cross. There would need to be a footpath crossing of the conveyor, these have been provided elsewhere and I am satisfied that it is possible to provide a safe crossing to maintain the right of way. Although it is clear that there will be some disturbance to users of rights of way, this disturbance would be temporary and it should be balanced with the enhancement of rights of way through the restoration scheme and management period, and also with the need for the sand and gravel. Therefore this proposal is acceptable in terms of OSP R2. The increased permissive public amenity access offered by the restoration accords with OMWLP PE12.

- There is some confusion over the status of the site. The response from WODC states that the site is allocated in the OMWLP. It is not allocated: the marking of the site on Inset Map 3 indicates only that there was once a resolution to grant planning permission for sand and gravel extraction on this site. Permission was not granted for that application, which dated from 1987. Therefore the site does not have any special status and should be considered against the development plan and other material considerations in the same way as any other unallocated site.

- Any further issues raised in the consultation process have been addressed at Annex 3 to this report.

Conclusions

- Although there would inevitably be effects on the local area, these have to be weighed against the need for sand and gravel and the fact that it can only be worked where it naturally occurs. Negative impacts can be controlled through the use of planning conditions.  The extraction period is temporary and following restoration the site would be managed for twenty years for nature conservation. Therefore, in the longer term the development could have positive effects on the local area. The development accords with development plan policies relating to need, traffic, flood risk and groundwater, ecology, archaeology, landscape character and amenity.

- Subject to a legal agreement, routeing agreement and conditions, this development is acceptable in terms of the development plan and there are no other material considerations that would justify its refusal. Therefore, it is recommended that it be approved.

- RECOMMENDATION

- It is RECOMMENDED that:

- (a)	subject to:

        - a legal agreement, covering (a) long term management of the site and its funding and funding of the wider Lower Windrush Valley Project though contributions per tonne of mineral extracted, including a permissive right of way and bird hide (b) bird management, (c) monitoring of routeing; and

        - a routeing agreement requiring that traffic to and from the development use only the approved route through Dix Pit, west along the B4449 and north along the  A415;

that planning permission for the development outlined in Application  07/0111/P/CM be approved subject to conditions to be determined by the Head of Sustainable Development but to include those matters set out in Annex 1 to this report; and

(b)	if the legal agreement referred to in (i) above is not completed within 10 weeks of the date of this meeting the Head of Sustainable Development be authorised to refuse the application on the grounds that it would not comply with OSP policies M1 or T8 in that:

(i) 	there would be no satisfactory provisions for the long term 	management of the restored site;

(ii)	that there would be bird strike risk;

(iii)	that the development would not be acceptable in transport 	terms as lorry movements would damage local amenity

- CHRIS COUSINS

- Head of Sustainable Development

- Environment & Economy

Background Papers:	Application for the extraction of sand and gravel with associated processing plant, silt ponds, conveyors and ancillary works. Restoration to wetland/reedbed and fishing. Extraction of basal clay to form hydrological seals and for the purposes of restoration on site

File 8.5/4102/2 located in Minerals and Waste Development Control Team area at Speedwell House, Speedwell Street, Oxford.

May 2008