TIC8 - page 4

ANNEX 2

SUMMARY OF CONCERNS
Comment
Number of Responses Raising Comment
Officer Response
Action Taken

Additional calming is needed for Quarry High Street.  If this road is left uncalmed traffic will use it as a rat run.
5
There are a large number of accesses along this road and the areas without accesses are used for parking.  If additional calming was installed on Quarry High Street it would not only be sporadic making it ineffective but would also reduce parking which residents do not want.
No additional calming is proposed.

The access from the eastern bypass should be either partially or fully closed.


7
The issue of closing the bypass access was rejected at the stakeholder meeting prior to the initial plans being drawn up.  Whilst there would be some benefits from closing the access it would also restrict access to legitimate users.  At no stage of the consultation has there been significant support to suggest residents would back such a proposal.
No action to be taken.

A lot of the pavements are too narrow and need widening to make it safer for pedestrians to walk through the area.
3
The roads which have narrow pavements such as Quarry High Street and Quarry Hollow also have narrow carriageways with on street parking.  If the footpaths were widened the parked cars would block the road.  However it may be possible to carry out some limited widening to alleviate current 'pinch points’.


Officers will look to increase footway width where feasible.

Additional humps are needed in Pitts Road and Trafford Road.
3
These roads are not part of the main routes through Quarry and therefore were not considered as needing calming.  There has been very little demand for these roads to be added to the scheme.  If traffic shifts to these roads as a result of the scheme additional measures could be added.
It is considered unlikely that vehicles will divert onto these roads.  However the situation will be monitored.

A one-way system should be created.
2
One of the 3 options proposed during the initial consultation was to create a one-way system.  However this was not supported (64% of responses were against it) so was not pursued.
No action to be taken.

A speed table should be provided at the Quarry Road/Old Road junction
3
This was not considered as part of the original proposals and has not been supported in large enough numbers since to consider it worthwhile.
This was only raised at formal consultation stage leaving insufficient time to include it in the scheme.  This could be reconsidered in the future.

Chicanes should be used to calm traffic instead of tables.
2
Using chicanes would result in a loss of parking which residents have previously said they do not want.  This is not a large enough response to suggest that view has changed.
No action to be taken.

Crossings should be provided for those taking children to school.
3


Installing crossings would not discourage rat running nor would it have much impact on speed, the two main problems identified by residents, during the initial consultation.  The junction tables can be used as crossing points.


Tactile paving will be provided at the tables to indicate that they can be used as uncontrolled crossing points.

The speed limits should be enforced using speed cameras.
1
Speed cameras would not discourage rat runners from using the area and are not suitable for calming such a large area.
This has not been pursued.

Erect ‘Local traffic only’ signs


1
These signs cannot be enforced and would likely have very little impact on traffic volume.  They would not reduce speeds.
This has not been pursued.

Ban HGV’s from entering Quarry.
1
Weight restrictions require a great deal of enforcement to be effective.  This type of vehicle makes up only a small proportion of rat running traffic so it would not be of much benefit.


This will not be pursued.

The table halfway down Trinity Road should be removed.
2
Although there has not been a great level of support for this we have encountered problems with drainage in the area.  As the road is very narrow it is difficult to drive particularly quickly along it.  In light of these facts we agree that this table should be removed.
The table halfway along Trinity Road will be removed.

Introduce parking restrictions
2
Residents of Quarry were opposed to the creation of a Controlled Parking Zone (CPZ) in the area when consultation was carried out.  We have been given no reason to believe that residents now wish to have restricted parking.


No action to be taken.

The access from the Green Road roundabout should be closed.
2
As stated in response to the idea of closing the eastern bypass, access closures were not considered following the initial stakeholder meeting.  There has been insufficient support shown during the consultation to consider this now.
This has not been pursued.

TIC_DEC2205R04.doc
TIC_DEC2205R04.doc

